IASC-VSC 2024

SUBJECT-SOP (Question paper with Answer Key)

Marks-100								Time: - 01:30 Hrs

- Variometer combines indication of vertical speed in both meters per second and feet per minute.

- Pre-flight briefing carried out, covering the complete sortie profile.

- After every six to seven landings or two hours of flying, aircrew should preferably take a short break to avoid fatigue.

- Ground crew to carry out tyre check after every landing for any creep or rubbing marks on the sides.

- Flying prohibited when OAT above 55 deg celsius.

- Aircraft approved for Day VFR operations only.

- Preflight meals / medical completed.  All participants are fully fit to fly.

- Flying prohibited with both doors open.

- Single magneto drop Max 300

- Propeller-FP 02 – 80 Fixed Pitch

- Flying prohibited when fuel used with more than 10 % alcohol.

- Wing surface area - 9.29 Sq meters.

- Aspect Ratio -11.8

- X COM VHF 760 has 99 memory channels of which one is for primary channel of VHF.

- Propeller is a two blade fixed pitch design.

- Pitot tube is attached to bottom side of right hand wing.

- Braking function is not available on the nose wheel.

- In cockpit leg length adjustment is provided for, on rudder pedals.

- Steep turns are to be carried out with initial speed of 100Kts.

- Centre of Gravity limits	-20% - 38% of MAC

- Rate of fuel flow is calculated from RPM and MAP (Manifold Pressure) measurements.

- Cooling air intakes are located left and right on the bottom part of the engine cover.

- Garmin AERA 500 GPS Equipped with a lithium ion battery that lasts up to 5 hours.

- Fuel Recommended-Unleaded Super Grade 93 or above Max 10 % alcohol.

- Normal Operating Speed VNO-108 Knots

- Lubrication System-Dry Sump Forced Lubrication

- Rudder is controlled via cables connected to pedals.

- Minimum Sink Rate Speed-58 Knots.

- Fuel Quantity. Displayed in percentage.

- Max Speed V FE 25 (Flaps 25°)-55 Knots.

- Best Climb Speed - 76 Knots

- During Take-off & Landing with flaps fully retracted.

- Landing roll at MTOW (SL) - 410 feet.

- Power on / off stalls are not to be carried out below 1500ft AGL.

- Absolute ceiling at MTOW- 6200 meters / 20,300 feet

- After pulling the activation handle, the main canopy opens in 3.2 seconds.

- Variometer starts registering when speed exceeds 36 Kts for more than 05 seconds.

- Parachute activation handle is located overhead between both pilots.

- Max useful loads -197.5 Kgs.

- Max Design maneuvering speed VA	-86 Knots.

- The propeller gear box provides a reduction ratio of 1: 2.27.

- Flying prohibited in heavy rainfall.

- Fuel flow Displayed in ltrs per hour

- Cruise Air Speed - 112 Kts

- Max Speed V FE 15 (Flaps 15°)-70 	Knots.

- Best L/D ratio Flaps 15°- 17: 1.

- Ignition System-Dual Breakerless Capacitor

- Elevator trim is electro-mechanical, driving a spring mechanism, with a common cockpit switch available to both pilots.

- Open full throttle and check engine RPM between 5300 – 5500 (at Sea Level & 25deg C) and not more than 5800 on ground.

- 48. Check brake serviceability every 200 meters.

Each Question Carries 05 Marks.

- Write down the emergency procedure of rough engine operation or failure in flight?

Ans. Rough engine operation or engine failure in flight.

(a) Ensure correct air speed 64 Kts

(b) Start analyzing terrain below

(c) Choose most appropriate site for landing out

(d) Provided engine failed aloft, react as follows.

(e) Ensure Master switch is in ON position

(f)  Magneto switches both set to ON

(g) Fuel valve open

(h) Attempt to restart the engine

(j)  If unsuccessful, begin with landing out procedure immediately

- Write down the emergency landing / landing off airport procedure?

Ans.	Emergency Landing / Landing Off Airport.

- Fuel valve OFF

- Master switch OFF

- Approach and land with extreme caution, maintain normal speed

- After having landed, leave the aircraft immediately

- Write down the emergency procedure of smoke in cockpit?

Ans. Smoke in Cockpit.

- Avionics OFF

- Disconnect the battery from the circuit

- Land as soon as possible

Note	In case of trouble breathing or visibility out of the cockpit has degraded severely due to smoke, open the cabin door and leave it hanging freely.  Flying with door open, do not, under any circumstances exceed 60 Kts.

- Write down the emergency procedure of icing/ Pneumatic Failure?

Ans: Icing / Pneumatic Instrument Failure.

(a) Turn back or change altitude to exit icing conditions

(b) Maintain VFR flight

(c) Set cabin heating ON

(d) Look out for signs of icing on Pitot tube

(e) In case of pneumatic instruments failure, use GPS information for reference

(f)  Plan to land at the nearest airport

(g) Manoeuvre the aircraft gently and keep flaps at Flaps ‘0’ position

- Write down the checks carried out for vital action downwind?

Ans. Vital Actions Downwind

(a) Check displacement correct / aircraft flying parallel to runway

(b) Height 700 feet AGL / Speed 70 – 75 knots

(c) Engine parameters within limits. Call out –

(d) Oil Pr / Oil Temp / CT 1 & CT 2 / EGT 1 & EGT 2 / Fuel quantity

(e) Landing lights ON.  RT Call

56. Write down the stall recovery procedure.

Ans. Stall Recovery Procedure.

(a) Move control stick forward to reduce angle of attack.  Horizon 1/3rd from top of canopy for clean stall and horizon on top of canopy for stall with flaps

(b) Smoothly open full power (throttle lever to fully forward position)

(c) Speed approaching 50 Kts, resume horizontal flight.  Do not stress the aircraft when pulling out

(d) Care must be taken not to exceed 70 Kts / 55 Kts during recovery from stall with Flaps 15° / Flaps 25°

57. Write down the Vital Actions before take-off.

Ans.	Vital Actions Berfore Take Off.

- Check Parachute safety pin removed.

- Select flaps as required for take off (Flaps 1 or Flaps 2 depending on runway available and winds)

- Trimmer in neutral position

- Choke fully forward

- Fuel valve open, fuel quantity adequate

- Landing light ON

- Master / Avionics / Magneto switches ON

- Flight instruments serviceable.  Altimeter set to airfield elevation, back to QNH, note correction

- Engine parameters callout

- Oil Pr / Oil Temp / CT 1 & 2 / EGT 1 & 2 / Fuel Qty (%)

- Check full and free movement of controls, no fowling or grinding noise

- Check Harness tight and door closed and locked.  Confirm from copilot

- Check wind direction and speed

- Note time.  Give RT call for line up.

58. Write down the checks before Engine Start.

Ans.	Checks before Engine Start.

- Check fuel quantity sufficient for duration of flight

- Confirm Pitot cover removed

- Confirm Parachute safety pin removed

- Engage wheel brakes and apply parking brakes

- Check fuel valve open, fuel CB IN, fuel pump on (by sound)

- In case of cold start, select choke fully open by pulling choke lever fully back

- Select Avionics OFF and both magnetos ON

- Check area around and propeller area clear.  Take clearance from ground crew for start

- Engage starter button till engine starts (not more than 10 seconds, in one go

- Select Avionics switch ON

- Check oil pressure registering and within limits

- Set throttle to adjust RPM below 2500

- Select choke lever fully forward (Closed) while maintaining RPM

- Check Engine parameters normal.

59. Write down Switch Off procedure.

Ans.  Switch Off procedure.

- Close throttle to idle.  Allow engine to cool down for 01 minute

- Parking Brakes ON.  Check aircraft not moving forward, give clearance to position chocks

- RT call

- Engine cool time over, select all green switches off

- Avionics OFF.  Both magnetos OFF.  Master switch OFF

- Fuel Pump CB OUT / Fuel shut off valve close

- Release parking Brakes

- Insert Parachute safety pin

- Exit aircraft and position pitot cover

60. Write down the Internal checks before Spin.

Ans. Internal checks before Spin.

- Height sufficient for recovery (Minimum 2500 ft AGL)

- Airframe configuration (Clean)

- Engine Parameters within limits

- Location, sufficient sector length available