IASC – VSC – 2023

VIRUS – SOP

PART – A : FILL IN THE BLANKS

1.	The thickness of cockpit wind shield glass is-2mm

2.	Undercarriage is robust tricycle type

3.	The propeller diameter of virus SW 80 aircraft is -1650mm

4.	The gearbox reduction ratio is 1:2.27

5.	Cylinder heads are cooled by Liquid cooling system

6.	Repacking of parachute is to be done after every -06 months

7.	ROTAX 912A is equipped with dry sump forced lubrication.

8.	The main undercarriage is equipped with disc type brakes.

9.	Elevator trim is operated-elecrto mechanical

10.	The main canopy of parachute opens in 3.2 sec.

11.	The Indian name of VIRUS SW 80 is GARUD.

12.	The fuel tank capacity is 50 Ltrs.

13.	The weight of battery is 1.62 kg.

14.	Parachute and Rocket system is installed behind right seat.

15.	The main wheel brakes are hydraulically operated.

16.	The flap lever has three position.

17.	Range of Tachometer is 0-7000 rpm.

18.	Wing surface area 9.29 sq metres.

19.	Aspect ratio of VIRUS SW-80 aircraft is 11.8.

20.	Oil capacity is 3 ltrs.

21.	Maximum baggage weight is 20 kgs.

21.	Max load per seat is 110 kgs.

22.	Compression ratio are 9:1.

23.	Maximum EGT is 900 deg C.

24.	Magneto drop check at 4000 rpm.

25.	Single magneto drop max is 300 rpm.

26.	Max difference in both magneto drop is 115 rpm.

27.	Best climb speed is 76 knots.

28.	Cruise air speed is 112 knots.

29.	Caution range of Tachometer is 5500-5800.

30.	Normal oil temperature is 90-110 deg C.

31.	Max oil pressure is 6 bar.

32.	Maximum speed for all operation is 135 knots.

33.	Normal operating speed is 108 knots.

34.	Stall speed at flap 0° position is 43 knots.

35.	Vertical fin area is 1.10 sq metre.

36.	Normal range of oil temperature is 90-110 deg C.

37.	Maximum permitted RPM is 5800.

38.	Flap position 2 is 25 deg C.

39.	Standard empty weight is 275 Kg.

40.	Best climb rate at 100 Kts 800ft per minute.

41.	Stall speed at flap 25° is 35 knots.

42.`	Minimum RPM in Tachometer is 1600

43..	Rate of fuel flow is calculated from RPM & MAP.

44.	Useable fuel quantity is 48 Ltrs.

45.	Location of fuel shutoff valve is in cockpit.

46.	Fuel hoses are protected with glass and silicon rubber.

47.	Lateral movement of control stick affects Aileron movement.

48.	Rudder is controlled via cables connected to pedal.

49.	The cabin has 2 fixed seats with H type safety harness.

50.	All composite parts are made of glass,carbon foam, fabric,Kevlar and  acrylic paint.

51.	VIRUS SW- 80 AC is manufactured by Pipistral,Slovenia,Europian Union.

52.	VIRUS SW- 80 AC is powered by Rotax 912 A engine.

53.	VIRUS SW- 80 AC ships with H type safety belt attached to the fuselage at three mounting points.

54.	Wing span of VIRUS SW- 80 AC is 10.5 metres.

55.	Cockpit windshields, doors are made of 2 mm anti UV GE Tinted Lexan.

56.	Cabin ventilation is achieved by vents provided on doors and upper wind shields.

57.	Leg length adjustment is provided on rudder pedals.

58.	All engine and flight control operating levers are located in cabin.

59.	Parachute and rocket system is installed behind right seat for emergency rescue.

60.	Location of parachute activation handle is on the back wall, between both the pilots.

61.	Undercarriage of VIRUS SW- 80 AC is robust tricycle type with two main wheels.

62.	The main undercarriage is equipped with disc type brakes.

63.	All wheel tyres are tube type in VIRUS SW- 80 AC

64.	Wheel brakes are hydraulically operated.

65.	Braking function is not available on nose wheels.

66.	The grade of hydraulic oil used in brakes system is DOT-3, DOT-4

67.	Elevator trim is electro mechanically operated.

68.	Flaperons are installed at the trailing edge of each wing .

69.	Two different control levers are used for flaps and ailerons.

70.	Lateral movement of the dual flight control stick affects the Aileron movement of flaperons.

71.	GRS rocket charged parachute rescue system is installed behind the right seat for emergency rescue.

72.	After pulling the activation handle the main canopy opens in 3.2 secs.

73.	The PRS handle is secured with safety pin.

74.	Repacking of parachute is to be done at every half year.

75.	Minimum height required for activation of PRS system is 300 fts.

76.	The minimum force required for activation of PRS handle is 11 kg.

77.	Fire extinguisher is a polyfoam AFFF compound.

78.	The propeller diameter of VIRUS SW- 80 AC is 1650 mm.

79.	Propeller is a two blade fixed pitch design.

80.	Propeller drive is achieved from the central cam shaft through an integrated reduction gear box.

81.	The reduction gear box provides a reduction ratio of 1:2.27.

82.	VIRUS SW- 80 AC is powered by ROTAX 912 A(80HP) 4 stroke 4 cylinder horizontally opposed twin carborated, spark ignition,single central cam shaft.

83.	Power plant of VIRUS SW- 80 AC is equipped with liquid cooling system.

84.	Rate of fuel flow is calculated from RPM &MAP measurements.

85.	Usable fuel capacity is 48 Ltrs in VIRUS SW- 80 AC.

86.	Cylinder heads are cooled by liquid.

87.	Cylinder walls are cooled by Ram air.

88.	Engine oil pump is driven by Cam Shaft.

89.	Hot oil after lubricating the engine is cooled by Radiator.

90.	The weight of batteries in VIRUS SW- 80 AC is 1.62 Kg.

91.	Battery is capable of operation within a temperature range of                           -30 deg C to +60 deg C

92.	Maximum operating altitude of battery is 5000 mts.

93.	Range of tachometer is 0-7000 rpm.

94.	RPM display turns red when engine RPM is above 5800.

95.	Engine Hour Totaliser starts counting engine hours when RPM exceeds 700.

96.	If the oil pressure rises above 94.3 psi displayed value becomes red with HIGH warning.

97.	Max -ve load factor of VIRUS SW-80 A/C is -2G.

98.	Take Off and landing of VIRUS SW-80 is prohibited with flaps fully retracted.

99.	The Yellow paint limit in ASI is 108-135 knots.

100.	Sink rate at 50 Kts is 440 feet per minute.