NCC DIRECTORATE –TN P &AN

2 TN AIR SQN FLY NCC

FLYING EXAM

SOP TEST

MARKS: 30

1.………… is recommended to have propeller life.

a)Painting   b) regular cleaning  c) periodic varnishing

2.Engine mounting bolts are totally ………. In number

a)5        b)6         c) 4

3.In case of prolonged climb, check & call out oil temp, pressure, coolant temp every …………

a) 400ft      b) 500ft        c) 300ft

4. Dry sump lubrication system provides lubrication in all flight conditions up to a max climb angle of……..

a) 45 deg     b) 40 deg      c) 50deg

5. The maneuvering speed of ML is…….

a) 60mph      b) 80mph        c) 70mph

6. Never exceed speeds of the a/c is ………….

a) 110mph   b) 100mph         c) 120 mph

7. Do not operating the a/c under sustained –g more than ……..

a) 2sec         b) 5sec              c) 3sec

8. Dry weight of the engine is 127 lb.

a) true          b) false

9. Flaps in zenair have been drooped to ensure wing root stall first.

a) True           b) false

10. The best angle of climb speed is ……..

a) 40mph      b) 30mph          c) 35mph

11. Displaced threshold will be at 1/4th way up the runway.

a) true            b) false

12. Zen air is capable of sustaining low flight at low speed due to presence of flaps.

a)true            b)false

13. The dry weight of the engine is……

a) 55.2 kg       b) 57.1kg       c) 57kg

14. Design load factor is………..

a) +3G to -6G      b) -6G to +3G      c) +6G to -3G

15. Carry ground run at least …………. a week incase a/c is not flying.

a) twice        b) once       c) thrice

16. In case of prolonged taxing check serviceability of brakes every ………

a) 250mts       b) 200mts      c) 100mts

17. The rotating propeller acts like a …………

a) torque         b) gyro            c) rotating wheel

18. The glide ratio during a forced landing is ……….

a) 7.9:1          b) 7.8:1       c) 7:8.1

19. Trim a/c for best L/D at ……

a) 45mph        b) 35mph       c) 30mph

20. Flaps extended speed range …………

a) 30 to 60mph      b) 40 to 70mph     c) 35 to 60mph

21. Cover the wind screen sheet with water proof cover to prevent hazing & corrosion.

a)true                b)false

22. Ensure propeller position is ……… to the ground.

a) Perpendicular      b) parallel      c) straight

23. Fuel consumption at max continuous rating …..

a) 23lt/hr          b) 22.6lt/hr          c) 22.7lt/hr

24. The crankshaft to propeller shaft ratio is ………..

a) 22:1             b)2.27:1              c)2:27.1

25. In order to stall the a/c solo the height shall not be below ……

a) 3000’            b) 4000’            b) 2000’

26. Check oil pressure registering & rises within ……. to min 30 psi during the warm start.

a)5sec              b)10sec              c)20sec

27. Rudder deflection is ………..

a) 23+ 2deg       b)23+/-  2deg      c)23+/-  1deg

28. Stalling speed of the a/c is………

a) 30mph           b) 35 mph            c) 40mph

29. Flying operations in Zen air must be carried out in……

a)  Night only    b) Daylight only     c) VFR conditions only

30. In forced landing, the low key is reached at the height of…………

a) 1500’AGL   b)2500’AGL    c)3500’AGL

GSK TEST

Marks:70

PRINCIPLES OF FLIGHT

1.…………… is the rate of change of position.

a)Acceleration          b) speed             c) motion

2.The stability about the longitudinal axis is known as……………

a)Lateral stability     b) normal stability      c) longitudinal stability

3.The angle of glide is least when L/D is……………

a)Minimum              b) maximum

4.The ……………… of an aircraft is that attitude beyond which the a/c will not climb.

a)Service ceiling       b) absolute ceiling         c)ceiling

5.Ram effect is efficient at low speeds.

a)True               b) false

6.Ozone is found in the ……………..

a)Ionosphere             b) stratosphere               c) exosphere

7.Volume of given mass of a perfect gas is inversely proportional to the pressure, provided temp remains constant,  is the condition of…………..

a)charles law              b) boyles law                  c) gas law

8.Position of the CG depends upon the angle of attack .

a)True                         b) false

9.In a turn , the total lift produced by the wings is always greater than the  A.U.W.

a)True                          b) false

10.A slip stream is produced which improves rudder and elevator control at low speeds in a turbo prop engine.

a)True                          b) false

AERO ENGINES

11. An ……………. Is a device wherein one form is converted into another form.

a)Engine              b) heat engine

12. The rotatory movement is utilised to transmit the power of the engine for ………

a)Thrust                b) propulsion              c) lift

13. Primary object of lubrication is …………………..

a)Cooling               b) reduce friction        c) dissipation of heat

14.Magneto is a special type of …………….. driven by the engine.

a)Electric generator       b) battery          c) generator

15. The distance a propeller moves forward in one revolution under conditions of no thrust is called ……………

a)Geometric pitch              b) experimental pitch

16.When used to drive a helicopter rotor a turbo prop engine is called ………… instead of turbo prop.

a)turbo fan                       b) turbo shaft                 c) turbo jet

17. the forward distance travelled by a propeller in one revolution is known as pitch.

a) true                      b) false

18.  in TSI, the slip indicator applies the ……… principle to indicate lateral level while the aircraft is at straight & level flight.

a) gyroscopic         b) pendulum

19. a mixture of ……………. Is used as a coolant for liquid cooling.

a) water & ethylene     b) water & ethylene glycol  c) water & glycol

20. in a dry sump system the pressure relief valve removes all coarse materials.

a) true                     b) false

AIRCRAFT COMPONENTS

21…………………. Have deep wings spars and thick wings.

a) monocoque           b)cantilever monoplane            c)braced monoplane

22. …………… is the main body of the aircraft to which all the other          components are attached.

a)main plane             b)fuselage                                 c)wings

23. ……………... are attached to the wings and are generally on the outside at the trailing edge.

a)flaps                       b)aileron                                    c)slats

24. The …………….. are the main lift giving components.

a)flaps                       b)wings                                      c)aerofoil

25. In ……………… the skin sheet metal is of sufficient strength and rigidity, a single sheet can be used to cover the wing without the actual support or spars.

a)stressed skin                 b)monocoque            c)D-Spar construction

26. In order to provide a control for reducing and regulating the air speed…………. have been designed

a)air brake                        b)brake                      c)trim

27. ………………. Fuselage is commonly used for small light weight aircraft

a)stressed skin                  b)monocoque construction  c)fabric covered

28. ……………. Is a small hinged surface forming part of the traling edges of the primary control surface

a)slat                                 b)trim                        c)flap

29. The fin of an aircraft is known as ………….

a)longitudinal stabiliser   b)vertical stabiliser    c)normal stability

30. A spar and rib combination covered with fabric is suitable for speeds upto……….

a)300kts                            b)350kts                    c)250kts

METEREOLOGY

31. …………. Is a phenomenon in which enormous amount of dust raised to a great height over a limited area reduced visibility to 200 mts or less

a)dust devil                      b)dust storm              c)dust raising wind

32. The principle weather chart used by vast majority of aviation is surface weather charts also known as ……….

a)surface chart                  b)synoptic chart

33. Visibility between 1000 – 2000 mts is ……………….

a)mist                                b)fog                         c)humidity

34. Sky divided into …………. Equal parts each known as one okta

a)4                                      b)8                            c)6

35. An altimeter reads .......... in colder air and ....... in warmer air.

a)higher & lower                  b)lower & higher           c) none

36. The ………. Charts the position of each reporting station is marked by a small circle with its three letter station named code along the side

a)upper wind chart    b)surface weather

37. The code form used for the transmission of radio sonal absorvation of pressure, temp, humidity and wind in the open air is called……..

a)synop code                     b)pilot code               c)temp code

38. …………… blows from land to sea.

a)land breeze                     b)sea breeze

39. The most common method of  measuring humidity is by determining the…………

a)celiometer          b)wet bulb temperature              c)transmissometer

40. Low clouds are called as ……………..

a)circus                  b)alto-stratus                   c)stratus

AIR NAVIGATION

41. ……… is to assit the pilot to make an approach for landing in poor visibility

a)approach lighting              b)aerodome lighting    c)air field lighting

42. Direction of the path of the aircraft with reference to the ground is called……..

a)track                                  b)track made good        c)drift

43. ……………. Is a vertical line joining the two poles of earth and its perpendicular to the equator and passing the centre of the earth

a)meridian                            b)earth axis                   c)isogonals

44. ……………. Is the angular difference between true north and magnetic north at a place

a)variation                            b)deviation

45. The direction in which the fore and aft axis of the aircraft is pointing is known as…………..

a) heading                            b)track

46. The direction of one point from the other in respect of the north is……

a)bearing                              b)drift                            c)heading

47. 1 nautical mile = ……………

a)5280 ft                              b)6080 ft                        c)6087.2 ft

48. 1 mile subtends an angle of 1deg at a distance of……………. Miles

a)60                                      b)90                               c)120

49. Kilometre is the length of 1 / 10000 part of the average distance between equator and either pole

a)true                                   b)false

50. Line cutting all the meridian at the same angle is called rhumbline

a)true                                   b)false

AIRMANSHIP

51. An…………… is a defined area of land or water including buildings and installation

a)aerodrome                      b)airfield                          c)airport

52. The ………………… usually works in a glass control cabin built at the top of the tower

a)approach controller   b)aerodrome controller

53.To house and park the aircraft ………. Are provided.

a)hangers                      b)blast pen

54. A taxi tract is constructed or a marked path, used by aircraft moving on ground at an airfield.

a)true                            b)false

55. When two air craft are approaching head on each is to alter heading to the right.

a)converging                b)diverging                  c)approaching head on

56. The air circuit is the airspace extending to…………… above airfield elevation

a)3000 ft                      b)8000 ft                       c)5000 ft

57. Two red balls on the signals mast and a double white cross in the signal square

a)glider flying in progress  b)parachute dropping in progress

58. A	black flag on the signal mast

a)no flying                    b)bad ground                c)landing prohibited

59. ………… is the health hazard for snow survival

a)typhus                        b)hypothermia             c)panic

60. Carbon monoxide poisoning is the health hazard of snow survival.

a)true                             b)false

AEROMODELING

61. ………… are found ideal for making propeller and wings of solid model

a)bass wood                            b)pine wood                 c)ply wood

62. …………. Is stronger than silver spruce but also heavier

a)birch                                    b)spruce                         c)bamboo

63. …………… are used for making various types of rubber model

a)rubber band                         b)rubber                          c)rubber strips

64. Glow plug is a type of  miniature engine used in aero models

a)true                                      b)false

65. two types of glow plug motors are jetex and pulse jet

a)true                                      b)false

66. Common piano wire is used for under carriages of aero models

a)true                                      b)false

67. Synthetic silk is useful since it dissolves out by the action of dope

a)true                                      b)false

68. Rubber bands are used for keeping various aircraft parts attached in respective positions of rubber models

a)true                                      b)false

69. Power flight is not advisable till the gliding test is carried out successfully

a)true                                      b)false

70. MLS

a)microwave landing system    b)microlight landing system