VIRUS SOP EXAM FOR BC

MAX MARKS-100

FILL IN THE BLANKS								     25X2=50

01.	VIRUS SW- 80 AC is manufactured by …………………………………

02.	Indian name of VIRUS SW- 80 AC is ………………………………….

03.	VIRUS SW- 80 AC is powered by ………………………… engine.

04.	VIRUS SW- 80 AC ships with……………… safety belt attached to the fuselage at 	…………….mounting points.

05.	Wing span of VIRUS SW- 80 AC is…………………….

06.	All composite parts of VIRUS SW- 80 AC are made of ………………………….

07.	Lateral movement of the dual flight control stick affects the …………….      ………movement of flaperons.

08.	Cockpit windshields, doors are made of …………………………..

09.	Cabin ventilation is achieved by …………………………

10.	Leg length adjustment is provided on …………………………

11.	All engine and flight control operating levers are located in………………..

12.	Parachute and rocket system is installed behind……………………for emergency rescue.

13.	Location of parachute activation handle is………………………..

14.	Undercarriage of VIRUS SW- 80 AC is ……………………… with two main  wheels.

15.	The main undercarriage is equipped with ……………………..brakes.

16.	All wheel tyres are……………….type in VIRUS SW- 80 AC

17.	Wheel brakes are……………….operated.

18.	Braking function is not available on ……………………… wheels.

19.	Flaperons are installed at the ………….. of each wing.

20.	.…………………levers are used for flaps and ailerons.

21.	…………………rocket charged parachute rescue system is installed behind the right seat for 	emergency rescue.

22.	After pulling the activation handle the main canopy opens in………………….secs.

23.	The propeller diameter of VIRUS SW- 80 AC is……………………….

24.	Cylinder heads are cooled by…………………

25.	Cylinder walls are cooled by………………………..

DESCRIPTIVE TYPE QUESTIONS							         5x10=50

01. Write down checks before Engine start

02. Write Magneto drop check  procedures.

03. Write emergency procedures during Engine fire on flight

04. Write circuit pattern followed by VIRUS SW 80 aircraft

05. Write emergency procedures during Engine failure during take off

ANSWER KEY

FILL IN THE BLANKS								     25X2=50

01.	VIRUS SW- 80 AC is manufactured by PIPISTRAL,SLOVENIA

02.	Indian name of VIRUS SW- 80 AC is GARUD

03.	VIRUS SW- 80 AC is powered by ROTAX 912A engine.

04.	VIRUS SW- 80 AC ships with H type  safety belt attached to the fuselage at 	3 mounting points.

05.	Wing span of VIRUS SW- 80 AC is 10.5 metres.

06.	All composite parts of VIRUS SW- 80 AC are made of glass,carbon foam,fabric,Kevlar and acrylic paint

07.	Lateral movement of the dual flight control stick affects the aileron      ………movement of flaperons.

08.	Cockpit windshields, doors are made of  2mm anti UV GE tinted Lexan

09.	Cabin ventilation is achieved by special vents provided

on doors.

10.	Leg length adjustment is provided on Rudder pedals

11.	All engine and flight control operating levers are located in cabin

12.	Parachute and rocket system is installed behind……………………for emergency rescue.

13.	Location of parachute activation handle is………………………..

14.	Undercarriage of VIRUS SW- 80 AC is ……………………… with two main  wheels.

15.	The main undercarriage is equipped with ……………………..brakes.

16.	All wheel tyres are……………….type in VIRUS SW- 80 AC

17.	Wheel brakes are……………….operated.

18.	Braking function is not available on ……………………… wheels.

19.	Flaperons are installed at the ………….. of each wing.

20.	.…………………levers are used for flaps and ailerons.

21.	…………………rocket charged parachute rescue system is installed behind the right seat for 	emergency rescue.

22.	After pulling the activation handle the main canopy opens in………………….secs.

23.	The propeller diameter of VIRUS SW- 80 AC is……………………….

24.	Cylinder heads are cooled by…………………

25.	Cylinder walls are cooled by………………………..